Context:

Where does the data come from?

BeeOmonitoring:

Aim: measure the concentrations of pollutants (here heavy metals and pesticides) in the environment.

How: With bees! Bees gather pollen within a 1.5 km radius, we collect and analyse the pollen.

When: between 2017 and 2020 – 4 times a year (4 periods: P1, P2, P3, P4) -> 16 periods

Where: on 63 sites in Belgium and France - each with a unique ID

The playing ground of bees is occupied by complex land use patterns as illustrated by the Corine Land Cover (CLC):

In order to characterise the environment, we can use

- the surface of each type of land use

- the distance from the hive (centre of the circle) to each polygon.

You’ll need:

- A list of the references of the sites

- The land use on the different sites (as area of each CLC class)

- The distance from the centre to each polygon of each type

- Information about CLC classes

- The measured concentrations of heavy metals over time

- Characteristics of heavy metals (Maximum Residue Levels)

- The measured concentrations of pesticides over time

- Characteristics of pesticides (Maximum Residue Levels + type)

Geographical data:

- absSurfs.xlsx: contains the area (in m²) for all the sites (column A, “Site” [1]) for each class (columns B to AS) of the Corine Land Cover (CLC) level 3 classes. These classes are represented by a three-digit number (code_clc) and used as column header. The surfaces are calculated in a circle with a radius of 1500m from a point, hence they all sum up to pi*r².  [1 - 2]

Column A is the exhaustive list of the sites considered (63 sites).

-  distsOneSheet.xlsx: contains the distance between the centre of the circle and each polygon in the circle with radius of 1500m.

Columns are:

- Site: Reference of the site [link to 1]

- polyID: Unique ID of the polygon to which the distance is calculated, as referred in CLC

- classCLC: level 3 code_clc of the polygon

- dist: distance in meter from the centre of the circle to the polygon. This column contains 	63 0’s [3]

- clc-nomenclature-c.xls: contains three sheets with the names of the CLC classes (column “libelle_en” in each sheet) and their code_clc (code_clc_LEVEL, in each sheet) for the three levels of classification.

Concerning the classification levels: the smaller the number of the level, the coarser the classification.

The code_clc adapts accordingly: level one: one digit, level two: two digits, level three: three digits.

Exemple:

Level1:

Code_clc_level_1 = 2 -> Agricultural areas

Code_clc_level_2 = 22 -> Permanent crops

Code_clc_level_3 = 221 -> Vineyards

Hence, code 221 in the level 3, is within code 22 in the level 2 and within level 2 in the level 1. [4]

Heavy Metals:

For the heavy metals (HM), the measured concentrations at the sites are stored in the folder:

YEAR/HM/HM_YEAR_PERIOD.xlsx

where YEAR ∊ [2017,2020] and PERIOD ∊ {P1, P2, P3, P4}

Example: 2017/HM/HM_2017_P4.xlsx: concentrations for the 4th period of 2017.

In each of these files:

- only one sheet is present with name YEAR_PERIOD.

- column A: “REF./SUBSTANCE” gives the reference of the sites [link to 1]

- column B: PERIOD indicates YEAR/PERIOD

- all the following columns contain the concentration for each of the 7 heavy metals (headers) in mg/kg.

- a value of 0 means that the heavy metal has been measured at a concentration 0 mg/kg.

[5]

LMR.txt: Maximum Residue Levels for the 7 heavy metals. All concentrations are given in mg/kg. NA indicates that no MRL is not defined at the European level for that metal. [6]

Pesticides:

For the pesticides, the measured concentrations at the sites are stored in the folder:

YEAR/Pesticides/Pesticides_YEAR_PERIOD.xlsx

where YEAR ∊ [2017,2020] and PERIOD ∊ {P1, P2, P3, P4}

Example: 2017/Pesticides/Results_Pesticides_2017_P4.xlsx: results for the 4th period of 2017.

In each of these files:

- only one sheet is present with name YEAR_PERIOD.

- column A: “REF./SUBSTANCE” gives the reference of the site [link to 1]

- column B: PERIOD indicates YEAR/PERIOD

- all the following columns contain the concentration for each pesticide (in mg/kg), identified at least once in the period. Number of columns depends on the number of pesticides identified within one period.

- a value of 0 means that the pesticide has been measured at a concentration 0 mg/kg.

[7]

- pesticides.xlsx: gives information about each pesticide

Columns are:

- importName: name of the pesticides as present in the column headers

- name: name of the pesticide (differs from previous column only in two cases)

- statusEN: indicates whether the pesticide is authorised at the European level

- LMR: The Maximal Residue Limit: a concentration given in mg/kg. An empty cell indicates that no MRL is available for that pesticide.

- familyEN: the family to which the pesticide belongs

- typeEN: the type of pesticide (fungicide, etc…). More than one type can be indicated.

Note: the word pesticide is a generic term. Types of pesticides can be herbicides (used against weeds), fungicides (used against mushrooms), insecticides (used against insects), etc.

Note that this file contains more pesticides than the list of unique pesticides in the files with the measured concentrations.

[8]